- Lecture Notes on SIZING

"No amount of genius can overcome a preoccupation with detail"

Murphy's law

To estimate the time it takes to do a task, estimate the time you think it should take, multiply by two, and change the unit of measure to the next higher unit. Thus allocate 2 days to do a one-hour task.

The law of optimum sloppiness

For any problem there is an optimum amount of sloppiness we can use to solve the problem.

KISS: Keep it simple, stupid

Corollary:

"There are occasions when we must be sloppy or imprecise in our calculations, and there are times when we must be precise. The essence of engineering is to be only as complicated as you have to be, but you must also be able to get as complicated as the problem demands".

- Separation Tower Design

Distillation:

Absorption:

Extraction:

- Adsorption:

- Sizing Problem

# of stages

Type of column

Height, Diameter, Cost

Shell Thickness & weight

Utility requirements, Operating Cost

- Types of Equipment

Plate Columns				     Packed Columns

(Finite stage contactors)		(continuous contactors)

- Sieve Trays			Packing type

- Bubblecap				Liquid redistributer

- Valve Trays

- Downcomer

- Packed versus Plate Tower

- Packed Tower

- Diameter < 4 ft

- Cannot handle dispersed solids in feed

- No interstage cooling

- Limited operating range

- not suitable for large temperature variations

- cheaper to construct

- design database is poor

- cheaper if corrosive fluids are involved

- pressure drop is smaller (good in vacuum operation)

- McCabe – Thiele Diagrams

q=1

Eqm line y=αx1+α−1x

LV=LD+L=xD−yFyD−xF=RR+1

Rmin=xD−yFyD−xF

LVachial=R1+R

Preliminary Design of Columns

1. Column Pressure and Temperature

Reboiler temp  boiling point of heavy component

Condenser temp  boiling point of light component

Increasing column pressure:  increases both temp.

decreases relative volatility 
and hence make 	separation 
more difficult

Considerations:  Are utilities available at condenser and reboiler?

2.  Selection of key components

A

B

C

D

A

B

C

D

E

F

Most of D goes

overhead

light key

E

F

heavy key

Incr. BP.

Most of E goes in

bottoms

[Assume 99%]

3. No. of stages  (Fenske-Underwood-Gilliland Method)

Used in DSTWU

1.  Assume 99% LK goes overheard, 99% HK goes in bottoms.  All components lighter goes with LK.  Heavier goes with HK.

2.  Do a material balance on column.  Determine mole fraction of light key in Distillate, (xLK)D etc

3.  Penske equation

Nmin=logxLKxHKDxHKxLKBlogαLKHK

4. Underwood Equation

1−q=iαixFiαi−θ  :  solve for θ

LDmin+1=iαixDiαi−θ	Compute minimum reflux ratio

5.  Gilliland Correlation

N,R

Solve For

4.  Plate Efficiency and Column Height

See Perry for one correlation

Assume 50% if no info is available

∴Actual # of trays = No. of theoretical stages xplate efficiency

Tray Spacing = 24”

Smaller for tall columns

Height = 24”  x   # of trays

5.  Column Diameter

v=velocityft/s=Gas flow rate  ft3/sArea  ft2

ev=vapor density   lb/ft3

vapor flow = L + D			L = Reflux

D = Distillate

D

F

L

(L+F)

F-D

L+D

L+D

L

Typical Velocities (of Vapor Flow)

Atmospheric		3 ft/sec

Vacuum		6 – 8 ft/sec

Pressure		1 ft/sec

Care must be taken in vacuum operation to minimize Δp across trays.

6.  Utility Requirements

Qc≈V⋅Heat of condensation of OverheadQR≈L+D⋅Heat of vaporization of Bottoms

Auxiliary Equipment Needed for column

Absorbers & Strippers

Pure Gas

GGa

Pure solvent, L lbmoles/hr

x≈0

y≈0

y = mx

G lbmoles/hr

Gas + Solute

Solvent + solute

yin

xout

LmG ≈ 1.4    typical

Kremser Equation

Packed Tower Design

Empirical Correlations available for HETP

Height = # of stages  HETP

Diameter fixed by vapor velocity

See Perry for correlation

- flooding

- channeling

Heat exchanger sizing

Problem:

Given: Flow rate and inlet and outlet temperature of the stream to be heated or cooled

Compute: Type and area of heat exchanger, Utility requirements, Pressure drop.

References:

Peters and Timmerhaus, pp. 528-573

D.Q. Kern, Process Heat Transfer

Perry's Handbook

Types of Heat Exchangers

- Double pipe heat exchanger

- Shell and Tube

- Extended surface

- Coiled tube

- Air-cooled

Selection of Tubeside fluid

- Corrosive fluids

- Fluid with greater fouling tendency

- Fluid at higher pressure

- Less viscous fluid

Heat Exchanger Geometry

- Lengths: 8, 12 and 16 ft standard

- Tube dia: 3/4 or 1 inch

- Tube wall thickness: Depends on pressure

- Baffle spacing: ~ shell diameter

Utility Selection

- Cooling Medium   Cooling water   75-110 F Return at 115 to 125 F

- Chilled water   40 F

- Refrigerant  < 32  ( Freon, Propylene)

- Dowtherm   for higher temperature

- Waste heat boiler ( at higher temperatures)

Heating Medium

- Low pressure steam   0-15 psig 250-275 F

- Medium pressure steam 15-150 psig, 360 F

- High pressure steam  < 500 psig, 450 F

- Dowtherm   < 750 F

- Fused Salt  < 1100 F

- Direct Fire  > 450 F

Short cut methods for HX design

Assume countercurrent flow

- 3/4 in OD tubes, 8 ft length

- < 10,000 sq.ft. area per exchanger

- Assume 15-20 F min approach temp.

- If necessary optimize area by adjusting outlet temp of utility

- Use tables and graphs for U.

- Keep Q/A < 12,000 Btu/hr/sq.ft in reboilers

- For coolers use max water outlet temp permissible

- For air-coolers use 20 hp per 1000 sq.ft of area. Air inlet at 90 F. Temperatue approach 40 at outlet

Pipe Design

Factors:

- Diameter of pipe

- Wall thickness

Pipe diameter

Small dia ⇒  high Δp

Large dia ⇒ higher cost

See Perry for correlations

Use friction factor charts to estimate Δp

w=z2−z1+v22−v122gc+P2V2−P1V1+ΣF

Pipe Wall Thickness

Schedule =1000PsSs=2000tmDm

Typical Schedule # 40, 80

Nominal vs. Actual

Pumps: Pressure change in liquids

Theoretical Horse Power: (THP)

Computed from Mechanical Energy Balance

Brake Horse Power = THPEfficiency

1. Centrifugal Pumps

- 15-5000 gpm

- 500 ft max head

2. Axial Pumps

- 20-100,000 gpm

- 40 ft head

eg. Bike pumps

3. Rotary Pumps

- 1,500 gpm

- 50,000 ft head

4. Reciprocating Pump

- 10-10,000 gpm

- 1,000,000 ft head

NPSH : Net Positive Suction Head 1-2 m of liquid

[Pin – Pvp]

Pressure change in gases

- Fans

- Blowers

- Compressors

- Ejecters for vacuum

Single stage versus Multistage

Interstage cooling needed

Pressure Vessels

Includes:	Flash Drums, Reactors, Tanks, Column shell etc.

t=PRSE−GP+C

Structural Rigidity ⇒ Min wall Thickness

Flash Drums:		H/D≈2−3

5 min holdup time (liquid)

Diameter based on gas velocity

Δp<1 psiin flash drums

used for 	Reactors

Flash Drums

Feflux Drum

At low pressures and large volumes, use storage tanks

Chemical Reactors

Factors Affecting Choice

1)	no. of phases present

2)	Pressure

3)	Temperature

4)	Residence time

5)	Conversion

6)	heat effects

Specify

i)	Volume of reactor

ii)	geometry

iii)	heat transfer

iv)	agitation

v)	material of construction

1.  Homogenous Gas Phase

- multiple empty tubes in parallel

- fast reaction , 1 sec ras. Time

- strong heat effects

furnaces for endothermic

diluent for exothermic

small die for exothermic

2.  Homogenous Liquid Phase

- CSTR for low to med conversions, slow reactions

(better heat transfer)

- Plug flow for faster reactions, high conversion

- combination may also be used

3. Hetero – liquid/gas

- stirred vessels with baffles/agitation

- use gas velocity

0.2 ft/sec if gas is mostly absorbed

0.1 ft/sec if gas is 50% absorbed

0.05 ft/sec if gas is mostly not absorbed

4.  Liquid/Solid

- well-stirred CSTR

- slurry reactors

5.  Solid/gas

- packed types (solid not consumed)

- fluidized bed

- spouted bed

Materials of Construction

Carbon steel, most commonly used

- Not suitable for dilute acids or alkaline solutions

- Brine, salts will cause corrosion

- Not suitable at high or cryogenic temperatures

Stainless Steel

- Type 302, 304, 316 common

- Corrosion resistance

- High temperature strength

Copper		good for alkalies

Nickel clad steel:	Caustic materials

Glass lined steel:

Plastics:	Moderate temperatures <400°F and pressures

Teflon:

Low temperature

Liq. Propylene		-53°F			201 s.s.

Liq. Elthylene		-154°F		9% nickel steel

LNG (methane)		-258°F		9% nickel steel

LNG Nitrogen		-320°F		304 s.s.

Cost Factors

| Material | Relative Cost | Comments |
| --- | --- | --- |
| Carbon steel | 1 | Low cost, most widely used |
| 304 s.s. clad steel | 5 | Acids |
| 316 s.s. clad steel | 6 |  |
| 304 s.s. | 7 | High T applications Corrosion Resistant |
| 316 s.s. | 10 | High T applications Corrosion Resistant |
| Inconel | 13 | Chlorides |
| Hastelloy c | 40 |  |
| Plastics |  | Low Temp Applications Low Structural Strength |
| Ceramics |  | High Temp. |
| Glass |  | Lab systems, Fragile Corrosion resistant |